Feasibility  of using Bivalve  Molluscs  to Reduce  the Suspended  Solid Content in Shrimp Farm Effluents

Abstract

The shrimp industry is now constrained by problems linked with environment due Lo direct discharge of farm effluent to the water source  In treatment systems, some bivalves can be utilized to reduce the suspended solid content in farm effluents and the present study was carried out whether species of bivalves namely Meretrtx casta and Perna perna, can be used for this purpose in Sri Lanka, Different densities of bivalves (20 m", 40 m', 60 m-3 and 80 m" ), from each species were separately allocated in effluent water and suspended solids were measured after  6  hours.   Twenty bivalves from both species were separately subjected to  the effluence  water (2 m3)   at two different salinity levels (20 ppt and 30 ppt) and suspended solids were measured   al 6 hour intervals Two different volumes or water (I  m3   and 2 m ')were  allowed to be utilized by 20 bivalves from each species and suspended solids were measured at 2 hour intervals Jn  both species, with the increase in the number of individuals, the suspended solid concentrations reduced,  P  perna was found to be significantly more efficient  in reducing the suspended solids than M.   casta (P<.0 05)  At 30 ppt salinity, there was significant  difference  (P<O 05)  between  the  two  species  in  their  suspended  solid reduction ability  'l'he   species which was capable oJ  reducing suspended solids at both salinity levels was P pe1 na  M   casta showed a significantly lower (P<O 05) suspended solid reduction ability at  30 ppt salinity  Significantly higher (P<O 05)  reduction  in suspended solid concentrations for both species was observed when total volume used was 1m1_ However, after 30 hours of purification no significant difference (P>0.05) was found between the two volumes Results of this study revealed that bivalves such as P perna and M. casta can be utilized in effluent treatment systems and suspended solid concentration can be

reduced from 850 mg/rrr' to less than half of the concentration within a period or 24 hours using 20 bivalves in   I  m3  of water

Keywords 	:

Citation 		: G.L.S.N.   Liyanage, U Edirisjnghe, W.M.T.B.  Wanninayake