Assessment of Microbiological Water Quality in West Coast Sea Bathing Sites of Sri Lanka

Abstract

Globally, there is a growing concern in investigating microbiological water quality in sea bathing sites due to the increasing coastal pollution. This is more important for islands such as Sri Lanka, since coastal pollution has considerable socio-economic implications. Therefore, this study was conducted to assess the current status of water quality in two popular sea bathing sites, Mount Lavinia (Colombo district) and Moragalla (Kalutara district) and a reference site in Kalutara district, which is a non-bathing site. Weekly changes of seven water quality parameters of the selected sites were recorded for three months and microbiological tests were performed to detect fecal coliforms (FC) and fecal streptococci (FS) separately. The comparison of these parameters in all three sites during the experimental period was conducted by ANOVA using Minitab 16 software. Sanitary inspection was conducted in both sea bathing sites to identify the potential sources of fecal contamination. Finally, Beach Suitability Grades (BSG) was determined by combining categories obtained from sanitary inspection and microbial assessment. The results showed that temperature, dissolved oxygen and pH were in the standard range for healthy sea bathing sites whereas salinity, conductivity, turbidity and total dissolved solids were below the standard range throughout the experimental period. A significant difference was observed in temperature, salinity, total dissolved solids and electrical conductivity between the three experimental sites (P<0.05). Further, Mount Lavinia showed a significantly higher FC count compared to the reference site (P<0.05). Based on the FC count, BSG for both Mount Lavinia and Moragalla sea bathing sites were determined to be ‘very high’. However, based on FS count, the BSG for Mount Lavinia and Moragalla were ‘poor’ and ‘fair’ respectively. Hence, it is recommended to develop and implement strategies including environmental education, water quality monitoring procedures and beach management to maintain healthy sea bathing sites in Mount Lavinia and Moragalla.

Keywords	: Beach Suitability Grades, microbiological, physico–chemical, sea bathing sites, 			  water quality

Citation 	: A.M.T.B. Alahakoon, S.C. Jayamanne, M.S. Kurukulasuriya, and A.J.M. 			  Gunasekara