Assessment of  the current status of water quality and polution levels in Hikkaduwa, Galle and Hambantota fishery  harbours Sri Lanka : A comparative study (Dissertation)

Abstract

Fishery harbour in Sri Lanka has been polluting in many aspects from the recent past duet to irregular monitoring of pollutants. This study was carried out with the objective of assessing  the water quality and severity of the pollution level in three selected fishery harbours namely Galle, Hambantota, Hikkaduwa, located in the Southern coast  of Sri Lanka. This was carried out from May 2014 to July 2014 and sampling was done on monthly basis from three stations from each harbour. Dissolved oxygen, water temperature, pH, turbidity, conductivity, total dissolved solids and salinity were determined in situ using termometer, pH meter, turbidity meter, conductivity and collected water samples were subjected to laboratory analysis using standard methods for the examination of water and waste water (APHA 20th edition). The study identified the main comtributors of determination of harbour water quality are vessel generated discharges, industrial, domestic waste water, user generated pollutants and external sources as natural phenominon.  pH, DO, salinity, nitrate-N, nitrite-N,EC were within the acceptable standards in each harbour. Harbour water was characterized significantly high Biological Oxygen Demand (9.67 ± 0.70), (7.10 ± 0.43), (9.17 ± 0.25) mg/L, respectively and high Chemical Oxygen Demand (719.51 ± 161.63), (926.50 ± 93.50), (840.20 ± 104.80) mg/L , high and oil & grease content (13.1 ± 0.60), (17.90 ± 0.20), (17.80 ± 1.20) mg/L in Hikkaduwa, Galle, and Hambantota respectively exceeded the recommended level. MPN values of fecal coliforms 2400/ 100 ml) which  exceeded the recommended standard in Galle and  Hambantota and Hikkaduwa has 1600 / 100ml. Diatoms and copepods were dominant in plankton studies of each harbour. Results of this  study revealed that the  harbour has been degraded in many aspects and harbour is subjected severe oil pollution, organic pollution and microbial contamination.

Keywords 	:

Citation 	: Jayampathi, O. M. M. D.