Pollution of Negombo Lagoon through the drainage canal system -

A threat to the estuarine eco system

Abstract

The Negombo estuary is one of the most productive brackish water bodies located along the west coast of Gampaha district in Sri Lanka. It is recognized as an economically important coastal wetland and serves both for people and nature. Now this vital ecosystem is facing a number of problems including pollution. The objective of this study is to identify and assess the drainage canal system in the Negombo Municipal Council area and their contribution to estuary pollution. The methods used for the study are field observations and analysis of water quality of selected drainage canals. The selection of the canals for water quality testing was based on the capacity of the canal and the visually observed features. The duration for field observation was 6 months from May to December 2012 on weekly basis. Physico- chemical parameters of water of Hamilton canal and Diyahonda Ela (the two selected canals for further study) were tested fortnightly from Mid-August 2012 to Mid-November 2012. Three sampling locations were selected from each canal with 25 m intervals. BOD, TSS, Oil and grease, Total Nitrate, Total Nitrite, and Total Phosphate loads were estimated based on the tested measurements. The load estimation was done using simple mathematics.

According to the results obtained, the estimated total volume of all the canal discharges was 4.0 x 105 m3 per day whereas Diyahonda Ela carries an estimated volume of 8.4 x 104 (m3 /day) of water and Hamilton Canal carriesan estimated volume of 2.9 x 105(m3/day) of water. Based on the maximum water discharge rate, the Hamilton canal transports a total load of TSS -7898 t/year, BOD - 1743 t/year, oil and grease - 1683 t/year, total nitrate - 270 t/year where as Diyahonda Ela carries a total load of TSS - 1186 t/year, BOD - 665 t/year, oil and grease - 202 t/year, total nitrate - 67 t/year to the estuary. Accordingly, pollutant load estimation revealed that the Hamilton canal is the main canal that transports higher loads of TSS, BOD, oil and grease and Nitrate concentrations to the estuary.

Water quality analysis of Hamilton canal and Diyahondaela showed that temperature, Ph and BOD concentrations of both canals are within the permissible discharge limits whereas TDS, oil and greese, nitrate and phosphate concentrations exceeded the maximum permissible limits as imposed by the CEA standards for discharging water into coastal and marine environment. The total coliform count and E.coli count also in both canals exceeded the countable range. Hence the results of the study indicate that a significant amount of pollutants is discharged to the estuary through the drainage canal system that should be taken into consideration in making decisions regarding the management of Negombo estuarine eco system.

Keywords 	: Negombo lagoon, Drainage canal, pollution

Citation 	: G.D.S. Priyadarshika, N.D.K.Dayawansa and  S. Pathmarajah