Investigation of surface water quality of Karagan Lewaya in Hambanthota

aftermaths of Tsunami

Abstract

Karagan Lewaya is a seasonally hypersaline costal lagoon in the Hambantota District. Total area of the lagoon is approximately 835 ha, with The relatively small catchment area of 58 km2 and annual catchment discharge of 67 million m3. The Lewaya was severely  affected by Tsunami on 26th December,2004. It was reported that several dead bodies, vehicles and household items were washed into the lagoon. The study was targeted to determine the quality of surface water of the lagoon. The quality surface water samples obtained from lewaya was tested for several chemical and microbiological parameters. Present study of surface water quality shows high level of biochemical oxygen demand (BOD) which indicates high pollution levels of surface waters. The nutrient parameter do not indicate adverse conditions  in general and found to be near natural  conditions. However elevated nitrate levels were observed at some points within the lagoon. Aerobic plate counts of all water samples were very higher than the standard limits for potable water. Some were observed with the coliform and faecal coliform counts. E.coli was identified in large numbers from all tested water samples where there cannot be any E.coli present in potable quality water. There was no incidence of V. cholerae contamination in any of the sample while five out of six were tested positive for Salmonella spp. however consequent sampling and analysis did not find any positive identification for Salmonella. The study of chemical and microbiological parameters of surface waters indicate that Karagan lewaya is highly polluted.

Keywords	 : water quality, Tsunami, Hambantota

Citation	 : Samarasekere K.N. and Abeygunawardane S.I. (2016). Microbiological quality 			   assessment of beach sand in Uswatakeiyawa, Prithipura coastal area, Sri Lanka. 			   Proceedings of the University of Peradeniya International Research Sessions, 			   Sri Lanka, Vol. 20, 4-5 November 2016.pp 05.